SAMPAVE ENGINEERING SUITE

PROFESSIONAL PAVEMENT DESIGN REPORT

Name of Work:

High Traffic + Low CBR Demo

| Item | Detail |
| --- | --- |
| Client Name | Client DEF |
| Consulting Firm | Pavement Engineering Consultants |
| Report Identifier | RP-2-20260624 |
| Submission Date | 24-Jun-2026 |

========================================================================

IMPORTANT NOTICE: Submission package is decision-support documentation. Final field execution requires review and sign-off by a qualified pavement engineer.

========================================================================

Contents of this Report

| Section | Governing Reference |
| --- | --- |
| Executive Summary | Project summary and engineering disclaimer |
| Traffic / ESAL / MSA Analysis | IRC:37-2018 / AASHTO-1993 |
| Flexible Pavement Structural Design | IRC:37-2018 |
| Report Provenance and Traceability | Audit metadata; no engineering calculations |
| Assumptions Sheet | Poisson's ratios, moduli, and growth presets |
| Limitations of the Report | Engineering screening disclaimers |
| Engineer Review Checklist | Checklist verification list |
| Engineering Sign-Off and Seal | Reviewer signatures |

EXECUTIVE SUMMARY

This professional pavement design report presents the mechanistic and empirical design parameters established for the work 'High Traffic + Low CBR Demo'. The evaluation incorporates design traffic analysis, subgrade pavement layer profiling, and mechanistic safety screening. A conventional flexible pavement design has been developed for a design traffic of 96.52 MSA, resulting in a total pavement thickness of 780 mm. The design calculations have been prepared under Decision Support Mode guidelines.

Disclaimer: This Engineering Intelligence Review and report is a decision-support screening tool. Final design acceptance for construction field execution remains subject to independent review and sign-off by a qualified pavement engineer.

REPORT PROVENANCE AND TRACEABILITY

Generation Metadata

| Item | Detail |
| --- | --- |
| Report generated at | 2026-06-24 11:59:33 |
| Operator identifier | QA Validator |
| Project ID | 2 |
| Project work name | High Traffic + Low CBR Demo |
| Report path | validation_reports\scenario_2_report.docx |

IITPAVE Schema-History Selection Summary

| Item | Detail |
| --- | --- |
| Selection status | all_available_history_selected |
| Available candidate IDs | None |
| Selected history IDs | None |
| Unknown requested IDs | None |
| Included history records | 0 |
| Propagated diagnostic rows | 0 |
| Engineering calculations allowed | No |

Validation Warning Summary

No report-time validation warnings recorded.

Export And Import Provenance

Project exchange format: sampave.project_export v1.0.

Project import creates new local rows; source record IDs are preserved only as provenance.

Imported IITPAVE schema-history audit IDs are shown exactly as recorded and are not remapped.

Report Environment Metadata

Application: SAMPAVE v1.0.0.

Runtime: Python 3.14.4.

Platform family: Windows 11

This provenance section is read-only audit metadata. It does not extract IITPAVE strains, run mechanistic calculations, evaluate fatigue or rutting, make IRC compliance claims, or issue engineering recommendations.

TRAFFIC / ESAL / MSA ANALYSIS

SamPave Report - High traffic + low CBR

Design basis: IRC:37-2018 (cl. 4.6 MSA, Table 1 VDF, cl. 4.4 LDF). AASHTO ESAL reported for cross-reference.

Pavement Laboratory  •  Report Date: 24-Jun-2026

Project Information

| Item | Detail |
| --- | --- |
| Name of Work | High Traffic + Low CBR Demo |
| Client | Client DEF |
| Agency | Arcstone Infrastructure |
| Submitted By | QA Validator |

Inputs

| Item | Detail |
| --- | --- |
| Road Category | NH / SH |
| Terrain | Plain |
| Lane Configuration | Two-lane carriageway |
| Initial Commercial Traffic (A) | 3000 CVPD |
| Annual Growth Rate (r) | 7.5 % |
| Design Life (n) | 15 yr |
| VDF (F) — user / preset | 4.5 |
| LDF (D) — user / preset | 0.75 |

Computed Quantities

N = (365 · A · [(1+r)^n − 1]/r) · D · F · 10⁻⁶   (IRC:37 cl. 4.6)

| Quantity | Symbol | Value | Unit |
| --- | --- | --- | --- |
| Growth Factor | GF | 26.1184 | — |
| VDF used | F | 4.5 | — |
| LDF used | D | 0.75 | — |
| Cumulative Design Traffic | N | 96.52 | MSA |
| AASHTO ESAL (alias N × 10⁶) | — | 96,523,682 | ESAL |
| Traffic Category | — | Heavy (30–100 MSA) | — |

Reserved for Future Expansion

| Item | Status |
| --- | --- |
| Axle-Load Spectrum | Not provided |
| Weigh-In-Motion Records | Not provided |
| Traffic-Survey Source | Not provided |

References

| Code | Clause / Table | Title / Note |
| --- | --- | --- |
| IRC:37-2018 | cl. 4.6 | Guidelines for the Design of Flexible Pavements |
| IRC:37-2018 | Table 1 | Guidelines for the Design of Flexible Pavements |
| IRC:37-2018 | cl. 4.4 | Guidelines for the Design of Flexible Pavements |
| IRC:37-2018 | Plates 1-4 | Guidelines for the Design of Flexible Pavements |
| AASHTO-1993 | — | Guide for Design of Pavement Structures |
| AASHTO-T307 | — | Determining the Resilient Modulus of Soils and Aggregate Materials |

Cumulative MSA via IRC:37-2018 cl. 4.6. VDF/LDF from IRC:37 Table 1 / cl. 4.4 when not user-supplied. AASHTO ESAL = MSA × 10^6.

FLEXIBLE PAVEMENT STRUCTURAL DESIGN

SamPave Report - High traffic + low CBR

Design basis: IRC:37-2018 — Guidelines for the Design of Flexible Pavements (4th Revision).

Pavement Laboratory  •  Report Date: 24-Jun-2026

1. Project Information

| Item | Detail |
| --- | --- |
| Name of Work | High Traffic + Low CBR Demo |
| Client | Client DEF |
| Agency | Arcstone Infrastructure |
| Submitted By | QA Validator |

2. Design Inputs

| Item | Detail |
| --- | --- |
| Road Category | NH / SH |
| Design Life (years) | 15 |
| Initial Commercial Traffic (CVPD) | 3000 |
| Traffic Growth Rate (% / year) | 7.5 |
| Vehicle Damage Factor (VDF) | 4.5 |
| Lane Distribution Factor (LDF) | 0.75 |
| Subgrade CBR — 4-day soaked (%) | 2.5 |
| Resilient Modulus (user-supplied) | auto (from CBR) |

3. Cumulative Design Traffic

Per IRC:37-2018 cl. 4.6:

N = (365 · A · [(1 + r)^n − 1] / r) · D · F · 10⁻⁶

| Parameter | Symbol | Value | Unit |
| --- | --- | --- | --- |
| Initial Commercial Traffic | A | 3000 | CVPD |
| Annual Growth Rate | r | 0.0750 | — |
| Design Life | n | 15 | years |
| Lane Distribution Factor | D | 0.75 | — |
| Vehicle Damage Factor | F | 4.5 | — |
| Growth Factor [(1+r)^n−1]/r | — | 26.1184 | — |
| Cumulative Design Traffic | N | 96.52 | MSA |

4. Subgrade Resilient Modulus

Per IRC:37-2018 Annex E:

M_R = 10 · CBR              for CBR ≤ 5 %

M_R = 17.6 · CBR^0.64       for CBR > 5 %

| Property | Value | Unit |
| --- | --- | --- |
| CBR (4-day soaked) | 2.5 | % |
| Resilient Modulus (M_R) | 25.0 | MPa |

5. Suggested Pavement Composition

| Layer | Material | Thickness (mm) | Typical Modulus (MPa) |
| --- | --- | --- | --- |
| Bituminous Concrete (BC) | BC | 40 | 3000 |
| Dense Bituminous Macadam (DBM) | DBM-II | 190 | 3000 |
| Wet Mix Macadam (WMM) | WMM | 250 | 450 |
| Granular Sub-base (GSB) | GSB | 300 | 200 |
| TOTAL | — | 780 | — |

Design selected from IRC:37 catalogue reference: Skeleton - CBR 3-5%, MSA >= 50. Layer composition is a catalogue-style suggestion. Cross-check against IRC:37-2018 Plates 1–4 and run a mechanistic analysis (IITPAVE) for the fatigue and rutting limits in cl. 6.4 before adoption for construction.

6. Mechanistic Checks (IITPAVE)

| Check | Status |
| --- | --- |
| Bituminous-layer fatigue (IRC:37-2018 cl. 6.4.1) | IITPAVE unavailable - mechanistic workflow has not run. Use Structural Design Compute to run local IITPAVE or configure SAMPAVE_IITPAVE_EXE. |
| Subgrade rutting (IRC:37-2018 cl. 6.4.2) | IITPAVE unavailable - mechanistic workflow has not run. Use Structural Design Compute to run local IITPAVE or configure SAMPAVE_IITPAVE_EXE. |

7. Designer Notes

IRC:37 catalogue reference: Skeleton - CBR 3-5%, MSA >= 50 | Catalogue database incomplete — engineer verification required. | This catalogue entry is based on the legacy Phase-4 skeleton composition and has not been fully verified against the complete set of IRC:37 Plates. Engineer verification is required. | Subgrade CBR (2.5%) is below the minimum catalogue limit of 3.0%. Suggesting Plate 1 (CBR 3%) composition as baseline; borrow soil subgrade or stabilization is required.

ENGINEERING INTELLIGENCE REVIEW - Flexible Pavement Design

| Item | Detail |
| --- | --- |
| Engineering Screening Score | 85 / 100 |
| Risk Classification | Acceptable with minor review |

Design Compatibility Screening Alerts

| # | Engineering compatibility screening alerts |
| --- | --- |
| 1 | Engineering review required: Bottom pavement layer 'Granular Sub-base (GSB)' is too stiff relative to subgrade support (ratio 8.0 > 4.0). |

Engineering Review Remarks

• Layer stack is acceptable. Minor review of non-standard modular ratios or thicknesses suggested.

• Review is based on static pavement guidelines. Final designs must be mechanistically validated in IITPAVE.

Disclaimer: This Engineering Intelligence Review is a decision-support screening tool. Final design acceptance requires independent review and sign-off by a qualified pavement engineer. The Engineering Screening Score is a mathematical index based on typical ranges and does not constitute final safety approval or certification.

References: IRC:37-2018 'Guidelines for the Design of Flexible Pavements' (Indian Roads Congress, 4th Revision).

ENGINEERING INTELLIGENCE REVIEW

Design screening checks based on static layer modular ratios, thicknesses, and material tier compatibility.

ENGINEERING INTELLIGENCE REVIEW - Flexible Pavement Design

| Item | Detail |
| --- | --- |
| Engineering Screening Score | 85 / 100 |
| Risk Classification | Acceptable with minor review |

Design Compatibility Screening Alerts

| # | Engineering compatibility screening alerts |
| --- | --- |
| 1 | Engineering review required: Bottom pavement layer 'Granular Sub-base (GSB)' is too stiff relative to subgrade support (ratio 8.0 > 4.0). |

Engineering Review Remarks

• Layer stack is acceptable. Minor review of non-standard modular ratios or thicknesses suggested.

• Review is based on static pavement guidelines. Final designs must be mechanistically validated in IITPAVE.

Disclaimer: This Engineering Intelligence Review is a decision-support screening tool. Final design acceptance requires independent review and sign-off by a qualified pavement engineer. The Engineering Screening Score is a mathematical index based on typical ranges and does not constitute final safety approval or certification.

ASSUMPTIONS SHEET

The design is based on the following standard engineering assumptions and presets:

| Parameter Description | Assumed / Preset Value |
| --- | --- |
| Poisson's Ratio (Bituminous Layer) | 0.35 |
| Poisson's Ratio (Granular Base/Sub-base) | 0.35 |
| Poisson's Ratio (Subgrade Soil) | 0.35 |
| Poisson's Ratio (Cement Treated Base - CTB) | 0.25 |
| Poisson's Ratio (Cement Treated Sub-grade/Sub-base - CTS) | 0.25 |
| Design Traffic Growth Rate | 7.5 % |
| Subgrade Soil Behavior Modulus Relation | IRC:37-2018 Formulae (MR = 17.6 * CBR^0.64 for CBR > 5) |

LIMITATIONS OF THE REPORT

1. This pavement design report is prepared as a decision-support guide for the client and the design engineers. It does not constitute official government approval or official regulatory stamp.

2. The layer thicknesses, material properties, and traffic projections are calculated using subgrade soils and traffic data inputs supplied by the client. Any variations in site conditions, soil properties, or traffic loads must be verified in the field and the design updated accordingly.

3. Final execution of the pavement construction works requires a detailed site-specific verification, structural validation, and formal sign-off/approval by a qualified pavement engineer.

ENGINEER REVIEW CHECKLIST

Review checklists completed by the certifying pavement engineer:

| Verification Checklist Item | Status / Remarks |
| --- | --- |
| Review Workflow Status | Draft |
| Design Review Checklist Completed | NO |
| Input Verification Completed | NO |
| Traffic Assumptions Verified | NO |
| Material Assumptions Verified | NO |
| IITPAVE Verification Status | Not Verified |
| Final Reviewer Notes / Remarks | No remarks added. |

ENGINEERING SIGN-OFF AND SEAL

___________________________________________

Signature of Reviewing Engineer

Date: _____________________________________

Seal / Registration No: ____________________