COMBINED PAVEMENT-DESIGN REPORT

SamPave Report - Very high MSA case

Pavement Laboratory  •  Report Date: 22-Jun-2026

Project Information

| Item | Detail |
| --- | --- |
| Name of Work | Very High MSA Case Demo |
| Client | Client GHI |
| Agency | Arcstone Infrastructure |
| Submitted By | QA Validator |

Contents of this Report

| Section | Governing Reference |
| --- | --- |
| Traffic / ESAL / MSA Analysis | IRC:37-2018 / AASHTO-1993 |
| Flexible Pavement Structural Design | IRC:37-2018 |
| Report Provenance and Traceability | Audit metadata; no engineering calculations |

REPORT PROVENANCE AND TRACEABILITY

Generation Metadata

| Item | Detail |
| --- | --- |
| Report generated at | 2026-06-22 15:56:10 |
| Operator identifier | QA Validator |
| Project ID | 3 |
| Project work name | Very High MSA Case Demo |
| Report path | validation_reports\scenario_3_report.docx |

IITPAVE Schema-History Selection Summary

| Item | Detail |
| --- | --- |
| Selection status | all_available_history_selected |
| Available candidate IDs | None |
| Selected history IDs | None |
| Unknown requested IDs | None |
| Included history records | 0 |
| Propagated diagnostic rows | 0 |
| Engineering calculations allowed | No |

Validation Warning Summary

No report-time validation warnings recorded.

Export And Import Provenance

Project exchange format: sampave.project_export v1.0.

Project import creates new local rows; source record IDs are preserved only as provenance.

Imported IITPAVE schema-history audit IDs are shown exactly as recorded and are not remapped.

Report Environment Metadata

Application: SAMPAVE v1.0.0.

Runtime: Python 3.14.4.

Platform family: Windows 11

This provenance section is read-only audit metadata. It does not extract IITPAVE strains, run mechanistic calculations, evaluate fatigue or rutting, make IRC compliance claims, or issue engineering recommendations.

TRAFFIC / ESAL / MSA ANALYSIS

SamPave Report - Very high MSA case

Design basis: IRC:37-2018 (cl. 4.6 MSA, Table 1 VDF, cl. 4.4 LDF). AASHTO ESAL reported for cross-reference.

Pavement Laboratory  •  Report Date: 22-Jun-2026

Project Information

| Item | Detail |
| --- | --- |
| Name of Work | Very High MSA Case Demo |
| Client | Client GHI |
| Agency | Arcstone Infrastructure |
| Submitted By | QA Validator |

Inputs

| Item | Detail |
| --- | --- |
| Road Category | NH / SH |
| Terrain | Plain |
| Lane Configuration | Two-lane carriageway |
| Initial Commercial Traffic (A) | 8000 CVPD |
| Annual Growth Rate (r) | 8 % |
| Design Life (n) | 20 yr |
| VDF (F) — user / preset | 5.5 |
| LDF (D) — user / preset | 0.75 |

Computed Quantities

N = (365 · A · [(1+r)^n − 1]/r) · D · F · 10⁻⁶   (IRC:37 cl. 4.6)

| Quantity | Symbol | Value | Unit |
| --- | --- | --- | --- |
| Growth Factor | GF | 45.7620 | — |
| VDF used | F | 5.5 | — |
| LDF used | D | 0.75 | — |
| Cumulative Design Traffic | N | 551.20 | MSA |
| AASHTO ESAL (alias N × 10⁶) | — | 551,202,860 | ESAL |
| Traffic Category | — | Very Heavy (> 100 MSA) | — |

Reserved for Future Expansion

| Item | Status |
| --- | --- |
| Axle-Load Spectrum | Not provided |
| Weigh-In-Motion Records | Not provided |
| Traffic-Survey Source | Not provided |

References

| Code | Clause / Table | Title / Note |
| --- | --- | --- |
| IRC:37-2018 | cl. 4.6 | Guidelines for the Design of Flexible Pavements |
| IRC:37-2018 | Table 1 | Guidelines for the Design of Flexible Pavements |
| IRC:37-2018 | cl. 4.4 | Guidelines for the Design of Flexible Pavements |
| IRC:37-2018 | Plates 1-4 | Guidelines for the Design of Flexible Pavements |
| AASHTO-1993 | — | Guide for Design of Pavement Structures |
| AASHTO-T307 | — | Determining the Resilient Modulus of Soils and Aggregate Materials |

Cumulative MSA via IRC:37-2018 cl. 4.6. VDF/LDF from IRC:37 Table 1 / cl. 4.4 when not user-supplied. AASHTO ESAL = MSA × 10^6.

FLEXIBLE PAVEMENT STRUCTURAL DESIGN

SamPave Report - Very high MSA case

Design basis: IRC:37-2018 — Guidelines for the Design of Flexible Pavements (4th Revision).

Pavement Laboratory  •  Report Date: 22-Jun-2026

1. Project Information

| Item | Detail |
| --- | --- |
| Name of Work | Very High MSA Case Demo |
| Client | Client GHI |
| Agency | Arcstone Infrastructure |
| Submitted By | QA Validator |

2. Design Inputs

| Item | Detail |
| --- | --- |
| Road Category | NH / SH |
| Design Life (years) | 20 |
| Initial Commercial Traffic (CVPD) | 8000 |
| Traffic Growth Rate (% / year) | 8 |
| Vehicle Damage Factor (VDF) | 5.5 |
| Lane Distribution Factor (LDF) | 0.75 |
| Subgrade CBR — 4-day soaked (%) | 4 |
| Resilient Modulus (user-supplied) | auto (from CBR) |

3. Cumulative Design Traffic

Per IRC:37-2018 cl. 4.6:

N = (365 · A · [(1 + r)^n − 1] / r) · D · F · 10⁻⁶

| Parameter | Symbol | Value | Unit |
| --- | --- | --- | --- |
| Initial Commercial Traffic | A | 8000 | CVPD |
| Annual Growth Rate | r | 0.0800 | — |
| Design Life | n | 20 | years |
| Lane Distribution Factor | D | 0.75 | — |
| Vehicle Damage Factor | F | 5.5 | — |
| Growth Factor [(1+r)^n−1]/r | — | 45.7620 | — |
| Cumulative Design Traffic | N | 551.20 | MSA |

4. Subgrade Resilient Modulus

Per IRC:37-2018 Annex E:

M_R = 10 · CBR              for CBR ≤ 5 %

M_R = 17.6 · CBR^0.64       for CBR > 5 %

| Property | Value | Unit |
| --- | --- | --- |
| CBR (4-day soaked) | 4 | % |
| Resilient Modulus (M_R) | 40.0 | MPa |

5. Suggested Pavement Composition

| Layer | Material | Thickness (mm) | Typical Modulus (MPa) |
| --- | --- | --- | --- |
| Bituminous Concrete (BC) | BC | 40 | 3000 |
| Dense Bituminous Macadam (DBM) | DBM-II | 190 | 3000 |
| Wet Mix Macadam (WMM) | WMM | 250 | 450 |
| Granular Sub-base (GSB) | GSB | 300 | 200 |
| TOTAL | — | 780 | — |

Design selected from IRC:37 catalogue reference: Skeleton - CBR 3-5%, MSA >= 50. Layer composition is a catalogue-style suggestion. Cross-check against IRC:37-2018 Plates 1–4 and run a mechanistic analysis (IITPAVE) for the fatigue and rutting limits in cl. 6.4 before adoption for construction.

6. Mechanistic Checks (IITPAVE)

| Check | Status |
| --- | --- |
| Bituminous-layer fatigue (IRC:37-2018 cl. 6.4.1) | IITPAVE unavailable - mechanistic workflow has not run. Use Structural Design Compute to run local IITPAVE or configure SAMPAVE_IITPAVE_EXE. |
| Subgrade rutting (IRC:37-2018 cl. 6.4.2) | IITPAVE unavailable - mechanistic workflow has not run. Use Structural Design Compute to run local IITPAVE or configure SAMPAVE_IITPAVE_EXE. |

7. Designer Notes

IRC:37 catalogue reference: Skeleton - CBR 3-5%, MSA >= 50 | Catalogue database incomplete — engineer verification required. | This catalogue entry is based on the legacy Phase-4 skeleton composition and has not been fully verified against the complete set of IRC:37 Plates. Engineer verification is required. | Design traffic (551.20 MSA) exceeds the maximum catalogue limit of 150.0 MSA. Suggesting 150.0 MSA composition as baseline; mechanistic design must be verified in IITPAVE.

References: IRC:37-2018 'Guidelines for the Design of Flexible Pavements' (Indian Roads Congress, 4th Revision).

| Prepared By ______________________ | Approved By ______________________ |
| --- | --- |