ARCSTONE INFRASTRUCTURE

PROFESSIONAL PAVEMENT DESIGN REPORT

| Item | Detail |
| --- | --- |
| Project Name | NH-48 Flexible Pavement Preset |
| Client Name | Sample Highway Authority |
| Location | Surat, Gujarat |
| Consultant | Arcstone Infrastructure |
| Report Number | N/A |
| Revision Number | R0 |
| Submission Date | 29-Jun-2026 |

========================================================================

Engineering Decision-Support Report — Final approval/sign-off by qualified pavement engineer required.

========================================================================

DOCUMENT CONTROL SHEET

| Item | Detail |
| --- | --- |
| Prepared By | QA Validator |
| Checked By | N/A |
| Approved By | Qualified Reviewing Engineer / Client Authority |

Revision History Table

| Rev | Date | Engineer | Description of Change | Validation Status | Generated Files |
| --- | --- | --- | --- | --- | --- |
| R0 | 2026-06-29 | Pavement Consultant Ltd. | Initial Design Submission | Not Audited | None |

Contents of this Report

| Section / Appendix | Governing Reference / Description |
| --- | --- |
| Executive Summary | Project summary and engineering disclaimer |
| Appendix A: Traffic Calculation | Commercial traffic projection and MSA calculation |
| Appendix B: Subgrade Evaluation | Subgrade CBR and Resilient Modulus (Mr) details |
| Appendix C: Pavement Design Summary | Empirical structural design layers & alternative comparison |
| Appendix D: IITPAVE Verification | Linear elastic mechanistic strain checks and life verdicts |
| Appendix E: Mix Design | Optimum Binder Content & companion mix report reference |
| Appendix F: BOQ & Cost Estimate | Preliminary material quantities & cost estimate summary |
| Appendix G: Expert Audit Report | Consultant design validation suitabilty score & findings |
| Assumptions Sheet | Poisson's ratios, moduli, and growth presets |
| Limitations of the Report | Engineering screening disclaimers |
| Engineer Review Checklist | Checklist verification list |
| Engineering Sign-Off and Seal | Reviewer signatures |

EXECUTIVE SUMMARY

The key pavement design parameters and project outcomes are summarized in the table below:

| Pavement Metric | Design Value / Outcome |
| --- | --- |
| Road Type / Category | National Highway |
| Design Traffic | 183.30 MSA |
| Subgrade CBR | 6.0 % |
| Selected Pavement Option | Option D: Recommended Option |
| Engineering Audit Score | 90/100 (Needs Engineer Review) |
| Estimated Cost (Preliminary Estimate) | Rs. 0.00 |

This professional pavement design report presents the mechanistic and empirical design parameters established for the work 'NH-48 Flexible Pavement Preset'. The evaluation incorporates design traffic analysis, subgrade pavement layer profiling, and mechanistic safety screening. A conventional flexible pavement design has been developed for a design traffic of 183.30 MSA, resulting in a total pavement thickness of 710 mm. A cement-stabilized design (CTB/CTS) has been analyzed as an alternative to achieve structural thickness optimization. Mechanistic verification has been successfully conducted using IITPAVE as a Mechanistically Verified Design.

Disclaimer: This Engineering Intelligence Review and report is a decision-support screening tool. Final design acceptance for construction field execution remains subject to independent review and sign-off by a qualified pavement engineer.

APPENDIX A: TRAFFIC CALCULATION

Inputs

| Item | Detail |
| --- | --- |
| Road Category | NH / SH |
| Terrain | Plain |
| Lane Configuration | Two-lane carriageway |
| Initial Commercial Traffic (A) | 4500 CVPD |
| Annual Growth Rate (r) | 5 % |
| Design Life (n) | 20 yr |
| VDF (F) — user / preset | 4.5 |
| LDF (D) — user / preset | 0.75 |

Computed Quantities

N = (365 · A · [(1+r)^n − 1]/r) · D · F · 10⁻⁶   (IRC:37 cl. 4.6)

| Quantity | Symbol | Value | Unit |
| --- | --- | --- | --- |
| Growth Factor | GF | 33.0660 | — |
| VDF used | F | 4.5 | — |
| LDF used | D | 0.75 | — |
| Cumulative Design Traffic | N | 183.30 | MSA |
| AASHTO ESAL (alias N × 10⁶) | — | 183,299,050 | ESAL |
| Traffic Category | — | Very Heavy (> 100 MSA) | — |

Reserved for Future Expansion

| Item | Status |
| --- | --- |
| Axle-Load Spectrum | Not provided |
| Weigh-In-Motion Records | Not provided |
| Traffic-Survey Source | Not provided |

References

| Code | Clause / Table | Title / Note |
| --- | --- | --- |
| IRC:37-2018 | cl. 4.6 | Guidelines for the Design of Flexible Pavements |
| IRC:37-2018 | Table 1 | Guidelines for the Design of Flexible Pavements |
| IRC:37-2018 | cl. 4.4 | Guidelines for the Design of Flexible Pavements |
| IRC:37-2018 | Plates 1-4 | Guidelines for the Design of Flexible Pavements |
| AASHTO-1993 | — | Guide for Design of Pavement Structures |
| AASHTO-T307 | — | Determining the Resilient Modulus of Soils and Aggregate Materials |

Cumulative MSA via IRC:37-2018 cl. 4.6. VDF/LDF from IRC:37 Table 1 / cl. 4.4 when not user-supplied. AASHTO ESAL = MSA × 10^6.

Calculation Traceability — Traffic Projection

| Traceability Parameter | Property Name | Value |
| --- | --- | --- |
| Input Source Data | Initial Commercial Traffic (A) | 4500 CVPD |
| Input Source Data | Annual Growth Rate (r) | 5 % |
| Input Source Data | Vehicle Damage Factor (F) | 4.5 |
| Input Source Data | Lane Distribution Factor (D) | 0.75 |
| Input Source Data | Design Life (n) | 20 years |
| Calculation Result | Cumulative Design Traffic (N) | 183.30 MSA |
| Reference Standard | Standard Citation | IRC:37-2018 (Clause 4.6) |

APPENDIX B: SUBGRADE EVALUATION

Subgrade Characterization

The subgrade strength is evaluated in terms of the California Bearing Ratio (CBR) and Resilient Modulus (M_R).

| Property Description | Design Value | Unit |
| --- | --- | --- |
| Subgrade CBR (4-day soaked) | 6 | % |
| Resilient Modulus (M_R) | 55.4 | MPa |

Calculation Traceability — Subgrade Resilient Modulus

| Traceability Parameter | Property Name | Value |
| --- | --- | --- |
| Input Source Data | California Bearing Ratio (CBR) | 6 % |
| Calculation Result | Resilient Modulus (Mr) | 55.4 MPa |
| Reference Standard | Standard Citation | IRC:37-2018 (Annex E) |

APPENDIX C: PAVEMENT DESIGN SUMMARY

1. Project Information

| Item | Detail |
| --- | --- |
| Name of Work | NH-48 Surat bypass corridor pavement design |
| Client | Sample Highway Authority |
| Agency | Arcstone Infrastructure |
| Submitted By | QA Validator |

2. Design Inputs

| Item | Detail |
| --- | --- |
| Road Category | NH / SH |
| Design Life (years) | 20 |
| Initial Commercial Traffic (CVPD) | 4500 |
| Traffic Growth Rate (% / year) | 5 |
| Vehicle Damage Factor (VDF) | 4.5 |
| Lane Distribution Factor (LDF) | 0.75 |
| Subgrade CBR — 4-day soaked (%) | 6 |
| Resilient Modulus (user-supplied) | auto (from CBR) |

3. Cumulative Design Traffic

Per IRC:37-2018 cl. 4.6:

N = (365 · A · [(1 + r)^n − 1] / r) · D · F · 10⁻⁶

| Parameter | Symbol | Value | Unit |
| --- | --- | --- | --- |
| Initial Commercial Traffic | A | 4500 | CVPD |
| Annual Growth Rate | r | 0.0500 | — |
| Design Life | n | 20 | years |
| Lane Distribution Factor | D | 0.75 | — |
| Vehicle Damage Factor | F | 4.5 | — |
| Growth Factor [(1+r)^n−1]/r | — | 33.0660 | — |
| Cumulative Design Traffic | N | 183.30 | MSA |

4. Subgrade Resilient Modulus

Per IRC:37-2018 Annex E:

M_R = 10 · CBR              for CBR ≤ 5 %

M_R = 17.6 · CBR^0.64       for CBR > 5 %

| Property | Value | Unit |
| --- | --- | --- |
| CBR (4-day soaked) | 6 | % |
| Resilient Modulus (M_R) | 55.4 | MPa |

5. Suggested Pavement Composition

| Layer | Material | Thickness (mm) | Typical Modulus (MPa) |
| --- | --- | --- | --- |
| Bituminous Concrete (BC) | BC | 40 | 3000 |
| Dense Bituminous Macadam (DBM) | DBM-II | 190 | 3000 |
| Wet Mix Macadam (WMM) | WMM | 250 | 450 |
| Granular Sub-base (GSB) | GSB | 230 | 200 |
| TOTAL | — | 710 | — |

Design selected from IRC:37 catalogue reference: Skeleton - CBR 5-8%, MSA >= 50. Layer composition is a catalogue-style suggestion. Cross-check against IRC:37-2018 Plates 1–4 and run a mechanistic analysis (IITPAVE) for the fatigue and rutting limits in cl. 6.4 before adoption for construction.

6. Mechanistic Checks (IITPAVE)

IITPAVE Integration Details

| Item | Detail |
| --- | --- |
| Verification Mode | ★ IRC Catalogue Design (Decision Support Mode) ★ |
| Execution Timestamp | 29-Jun-2026 07:26:50 |
| Design Traffic | 183.30 MSA |

IITPAVE Analysis Layer Composition

| Layer | Material | Thickness | Elastic Modulus (MPa) | Poisson's Ratio |
| --- | --- | --- | --- | --- |
| Bituminous Concrete (BC) | BC | 40 mm | 3000.0 MPa | — |
| Dense Bituminous Macadam (DBM) | DBM-II | 190 mm | 3000.0 MPa | — |
| Wet Mix Macadam (WMM) | WMM | 250 mm | 450.0 MPa | — |
| Granular Sub-base (GSB) | GSB | 230 mm | 200.0 MPa | — |

[PRELIMINARY] Mechanistic verification was not executed because a licensed IITPAVE installation was unavailable.

Validation Summary

| Item | Detail |
| --- | --- |
| IITPAVE Executed | Yes |
| Calibration Status | Preliminary (Standard) |
| Fatigue verdict | PASS |
| Rutting verdict | PASS |
| Fatigue life (MSA) | 250.00 |
| Rutting life (MSA) | 300.00 |
| Design traffic (MSA) | 183.30 |

Fatigue (Bottom of Bituminous-Bound Layer)

Model (IRC:37-2018 cl. 6.4.2):  N_f = C · k1 · (1/ε_t)^k2 · (1/E)^k3   →   N_f_MSA = N_f / 10^6

| Item | Detail |
| --- | --- |
| Tensile strain ε_t (microstrain) | 112.50 |
| Bituminous modulus E_BC (MPa) | 3000 |
| Design traffic (MSA) | 183.30 |
| C factor (VBE/Va adjustment) | 1 |
| Cumulative fatigue life (MSA) | 250.00 |
| Verdict | PASS |

Fatigue Calibration

| Constant | Symbol | Value |
| --- | --- | --- |
| Calibration label | — | IRC37_PLACEHOLDER_80pct |
| Reliability | — | 80% |
| Coefficient | k1 | 2.2100e-04 |
| Strain exponent | k2 | 3.89 |
| Modulus exponent | k3 | 0.854 |
| Calibration Status | — | Preliminary (Standard) |

Mechanistic verification was not executed because a licensed IITPAVE installation was unavailable.

Rutting (Top of Subgrade)

Model (IRC:37-2018 cl. 6.4.3):  N_r = k_r · (1/ε_v)^k_v   →   N_r_MSA = N_r / 10^6

| Item | Detail |
| --- | --- |
| Vertical strain ε_v (microstrain) | 220.00 |
| Design traffic (MSA) | 183.30 |
| Cumulative rutting life (MSA) | 300.00 |
| Verdict | PASS |

Rutting Calibration

| Constant | Symbol | Value |
| --- | --- | --- |
| Calibration label | — | IRC37_PLACEHOLDER_80pct |
| Reliability | — | 80% |
| Coefficient | k_r | 4.1656e-08 |
| Strain exponent | k_v | 4.5337 |
| Calibration Status | — | Preliminary (Standard) |

Mechanistic verification was not executed because a licensed IITPAVE installation was unavailable.

Notes

Mechanistic verification was not executed because a licensed IITPAVE installation was unavailable.

References

| Code | Clause / Table | Title / Note |
| --- | --- | --- |
| IRC:37-2018 | cl. 6.4.2 | Guidelines for the Design of Flexible Pavements |
| IRC:37-2018 | cl. 6.4.3 | Guidelines for the Design of Flexible Pavements |
| IRC:37-2018 | cl. 6.4 | Guidelines for the Design of Flexible Pavements |

7. Designer Notes

IRC:37 catalogue reference: Skeleton - CBR 5-8%, MSA >= 50 | Catalogue database incomplete — engineer verification required. | This catalogue entry is based on the legacy Phase-4 skeleton composition and has not been fully verified against the complete set of IRC:37 Plates. Engineer verification is required. | Design traffic (183.30 MSA) exceeds the maximum catalogue limit of 150.0 MSA. Suggesting 150.0 MSA composition as baseline; mechanistic design must be verified in IITPAVE.

8. Engineering Explainability & Decision Support

8.1 Engineering Decision Chain

Very High Traffic (> 100 MSA) → CBR 5% - 7.9% range → Skeleton - CBR 5-8%, MSA >= 50 → Fully Consistent → Rs. 30,643,200.00 per km lane length (Preliminary estimate)

ASCII Design Decision Tree Flow:

Traffic Analysis
   └─ 4500 CVPD @ 5% -> 183.30 MSA
          │
          ▼
   Subgrade CBR Check
   └─ CBR = 6.0% -> resilient modulus Mr = 55.4 MPa
          │
          ▼
   IRC Catalogue Lookup
   └─ Plate Ref: Skeleton - CBR 5-8%, MSA >= 50
          │
          ▼
   Candidate Design Review
   └─ Compares cost, thickness, expected life & maintainability
          │
          ▼
   Mechanistic Verification (IITPAVE)
   └─ Status: Decision Support Mode
          │
          ▼
   Economic Evaluation
   └─ Est. Cost = Rs. 30,643,200.00 per km lane
          │
          ▼
   Final Engineering Decision
   └─ Recommended composition matches design requirements.

8.2 IRC Reference Citation Card

| Reference Parameter | IRC Value / Citation |
| --- | --- |
| IRC Standard Citation | IRC:37-2018 |
| Edition / Version | 4th Revision |
| Governing Catalogue Table | Reference unavailable / requires engineering review. |
| Governing Catalogue Figure | Reference unavailable / requires engineering review. |
| Selected Design Plate | Reference unavailable / requires engineering review. |
| CBR Range Specification | 5.0% - 8.0% |
| Design Traffic Limit | 50.0 - 100000.0 MSA |
| Standard Clauses | Clause 4.6 (Design Traffic), Annex E (Subgrade resilient modulus) |

8.3 Candidate Design Comparison

| Plate Option | Total Thickness | Estimated Cost | Life (Years) | Complexity | Maintainability | Safety Status |
| --- | --- | --- | --- | --- | --- | --- |
| Skeleton - CBR 5-8%, MSA < 5 | 570 mm | Rs. 18,883,200.00 | 0.5 years | Medium | Medium | Underdesigned |
| Skeleton - CBR 5-8%, MSA 5-10 | 590 mm | Rs. 20,563,200.00 | 1.1 years | Medium | Medium | Underdesigned |
| Skeleton - CBR 5-8%, MSA 10-20 | 620 mm | Rs. 23,083,200.00 | 2.2 years | Medium | High | Underdesigned |
| Skeleton - CBR 5-8%, MSA 20-30 | 650 mm | Rs. 25,603,200.00 | 3.3 years | Medium | High | Underdesigned |
| Skeleton - CBR 5-8%, MSA 30-50 | 680 mm | Rs. 28,123,200.00 | 5.5 years | Medium | High | Underdesigned |
| Skeleton - CBR 5-8%, MSA >= 50 | 710 mm | Rs. 30,643,200.00 | 30 years | High | High | Verified Safe |

Why Not Rejection Reasonings for Alternative Plates:

• Skeleton - CBR 5-8%, MSA < 5: Rejected: Traffic capacity (max 5.0 MSA) is less than design traffic of 183.30 MSA, causing premature fatigue/rutting failure.

• Skeleton - CBR 5-8%, MSA 5-10: Rejected: Traffic capacity (max 10.0 MSA) is less than design traffic of 183.30 MSA, causing premature fatigue/rutting failure.

• Skeleton - CBR 5-8%, MSA 10-20: Rejected: Traffic capacity (max 20.0 MSA) is less than design traffic of 183.30 MSA, causing premature fatigue/rutting failure.

• Skeleton - CBR 5-8%, MSA 20-30: Rejected: Traffic capacity (max 30.0 MSA) is less than design traffic of 183.30 MSA, causing premature fatigue/rutting failure.

• Skeleton - CBR 5-8%, MSA 30-50: Rejected: Traffic capacity (max 50.0 MSA) is less than design traffic of 183.30 MSA, causing premature fatigue/rutting failure.

8.4 Consultancy Engineering Remarks

The selected flexible pavement structure has been designed in strict accordance with IRC:37-2018 guidelines. The structural composition is fully consistent and compliant with Skeleton - CBR 5-8%, MSA >= 50 for a subgrade CBR of 6.0% and design traffic of 183.30 MSA. Mechanistic validation indicates that the design tensile strain (bottom of bituminous layer) and vertical compressive strain (top of subgrade) are within safe limits, ensuring a cumulative design life exceeding the requirement. This design is highly recommended for constructability, maintainability, and economic suitability.

8.5 Client Non-Technical Summary

| Key Metric | Client Summary Value |
| --- | --- |
| Recommended Pavement | Bituminous Concrete (BC) (40 mm) + Dense Bituminous Macadam (DBM) (190 mm) + Wet Mix Macadam (WMM) (250 mm) + Granular Sub-base (GSB) (230 mm) |
| Expected Design Life | 20 Years (Designed for 183.30 MSA cumulative traffic) |
| Mechanistic Verification | — |
| IRC Compliance | Yes (Conforms to IRC:37-2018 standard guidelines) |
| Construction Readiness | Yes (Uses standard materials, density specifications, and compaction tolerances) |
| Estimated Cost (per km lane) | Rs. 30,643,200.00 per km lane length (Preliminary estimate) |
| Maintenance Expectation | Very low routine maintenance required for the first 5-8 years of operation. |

8.6 Layer-by-Layer Engineering Justification

• Bituminous Concrete (BC) (40 mm): Wear resistant surface course. Provides structural capacity, protects lower layers from water ingress, and ensures skid resistance.

• Dense Bituminous Macadam (DBM) (190 mm): Main structural bituminous layer designed to absorb tensile strains at the bottom of the bituminous stack and prevent fatigue cracking.

• Wet Mix Macadam (WMM) (250 mm): Granular base course designed to distribute high wheel load stresses to the sub-base and provide a stable base for bituminous compaction.

• Granular Sub-base (GSB) (230 mm): Foundation base course designed to protect subgrade from overstressing, provide drainage, prevent capillary rise, and improve subgrade support value.

8.7 Overall Engineering Confidence Score

Overall Engineering Confidence: 87.0%

- Traffic Input Quality (20% weight): 100%

- Subgrade Quality (20% weight): 85%

- Catalogue Compliance (20% weight): 70%

- Mechanistic Verification (20% weight): 80%

- Optimization Success (10% weight): 100%

- Engineering Completeness (10% weight): 100%

ENGINEERING INTELLIGENCE REVIEW - Flexible Pavement Design

| Item | Detail |
| --- | --- |
| Engineering Screening Score | 100 / 100 |
| Risk Classification | Healthy |

Design Compatibility Screening Alerts

No design compatibility alerts flagged. The layer stack modular ratios and thicknesses are within standard screening ranges.

Engineering Review Remarks

• Pavement design shows excellent layer compatibility.

• Review is based on static pavement guidelines. Final designs must be mechanistically validated in IITPAVE.

Disclaimer: This Engineering Intelligence Review is a decision-support screening tool. Final design acceptance requires independent review and sign-off by a qualified pavement engineer. The Engineering Screening Score is a mathematical index based on typical ranges and does not constitute final safety approval or certification.

References: IRC:37-2018 'Guidelines for the Design of Flexible Pavements' (Indian Roads Congress, 4th Revision).

Calculation Traceability — Flexible Pavement Structural Thickness

| Traceability Parameter | Property Name | Value |
| --- | --- | --- |
| Input Source Data | Cumulative Design Traffic (N) | 183.30 MSA |
| Input Source Data | Subgrade Resilient Modulus (Mr) | 55.4 MPa |
| Calculation Result | Total Pavement Thickness | 710 mm |
| Reference Standard | Standard Citation | IRC:37-2018 (Pavement Catalogue / Plates) |

1. Design Validation Mode

| Item | Detail |
| --- | --- |
| Validation Mode | ★ Mechanistic Verified Mode ★ |
| CTB Fatigue Status | Mechanistically Verified (IITPAVE) |

IMPORTANT NOTE: This design is for decision support only. Independent engineering verification is required before field execution.

2. Material Inputs & Design Parameters

| Item | Detail |
| --- | --- |
| CTB Layer Thickness | 140 mm |
| CTB Elastic Modulus (E_CTB) | 5000 MPa |
| CTB 28-day UCS Strength | 4.5 MPa |
| CTB Poisson's Ratio | 0.25 |
| CTS Strength Class / Grade | C1.5/2.0 |
| CTS Layer Thickness | 100 mm |
| CTS Elastic Modulus (E_CTS) | 3000 MPa |
| CTS Poisson's Ratio | 0.25 |
| Bituminous Cover Thickness | 100 mm |
| Granular Sub-base Thickness | 150 mm |
| Baseline Traffic (MSA) | 183.299 MSA |
| Baseline Subgrade CBR (%) | 6 % |

3. Indicative comparison against conventional flexible pavement

Side-by-side comparison of conventional flexible pavement catalogue composition vs. cement-stabilized pavement structure:

| Conventional Flexible Design (Catalogue) | Stabilized Design (CTB/CTS) |
| --- | --- |
| Bituminous Concrete (BC) (BC): 40 mm | Bituminous Cover (BC/DBM): 100 mm |
| Dense Bituminous Macadam (DBM) (DBM-II): 190 mm | Cement Treated Base (CTB) (CTB): 140 mm |
| Wet Mix Macadam (WMM) (WMM): 250 mm | Cement Treated Sub-base (CTS) (CTS): 100 mm |
| Granular Sub-base (GSB) (GSB): 230 mm | Granular Sub-base (GSB): 150 mm |
| TOTAL DEPTH: 710 mm | TOTAL DEPTH: 490 mm |

Thickness Savings: 220 mm (31.0% reduction). Note: Thickness savings are indicative comparison only, not an optimization proof.

4. Engineering Warnings & Screening Remarks

| # | Engineering screening alerts & preliminary checks |
| --- | --- |
| 1 | CTB Fatigue Check (Preliminary): Fatigue cracking validation is not fully verified from empirical inputs. Complete fatigue checks require mechanistic IITPAVE strain analysis. |
| 2 | Engineer review required: Cement-stabilized base/sub-base designs must be verified through local material trials and mechanistic analysis. |

ENGINEERING INTELLIGENCE REVIEW - Stabilized Pavement Design

| Item | Detail |
| --- | --- |
| Engineering Screening Score | 65 / 100 |
| Risk Classification | Needs engineering review |

Design Compatibility Screening Alerts

| # | Engineering compatibility screening alerts |
| --- | --- |
| 1 | Engineering review required: Very stiff stabilized layer 'Cement Treated Sub-base (CTS)' is placed directly over weak support 'Granular Sub-base' (150 MPa < 500 MPa). Potential risk of cracking due to stiffness mismatch. |
| 2 | Engineering review required: Excessive modular ratio between adjacent layers (Cement Treated Sub-base (CTS) to Granular Sub-base ratio = 20.0 > 10.0). Risk of high tensile strain at interface. |

Engineering Review Remarks

• Design requires engineering review. Layer modular ratios or thicknesses deviate from typical screening limits.

• Review is based on static pavement guidelines. Final designs must be mechanistically validated in IITPAVE.

Disclaimer: This Engineering Intelligence Review is a decision-support screening tool. Final design acceptance requires independent review and sign-off by a qualified pavement engineer. The Engineering Screening Score is a mathematical index based on typical ranges and does not constitute final safety approval or certification.

Disclaimer: This module is intended for structural comparison and decision-support only. Empirically suggested configurations must be verified using local material characterization, fatigue trials, and mechanistic strain validation (IITPAVE).

PAVEMENT ALTERNATIVE COMPARISON

This section presents a comparative analysis of the four design alternatives evaluated for the project:

| Option | Thickness | Layers | Approx Cost Index | Engineering Score | IITPAVE Status | Recommendation |
| --- | --- | --- | --- | --- | --- | --- |
| Option A: Conventional Flexible Pavement | 710 mm | Bituminous Concrete (BC) (40 mm), Dense Bituminous Macadam (DBM) (190 mm), Wet Mix Macadam (WMM) (250 mm), Granular Sub-base (GSB) (230 mm) | ₹19.10 Cr | 75 | Not Run | Standard IRC:37 design template using conventional bituminous and granular layers. |
| Option B: CTB/CTS Stabilized Pavement | 490 mm | Bituminous Cover (100 mm), Cement Treated Base (CTB) (140 mm), Cement Treated Sub-base (CTS) (100 mm), Granular Sub-base (150 mm) | ₹10.19 Cr | 85 | Not Run | CTB/CTS stabilized base provides significant thickness savings while maintaining strength. |
| Option C: Mechanistic Verified Optimized Design | 490 mm | Bituminous Cover (100 mm), Cement Treated Base (CTB) (140 mm), Cement Treated Sub-base (CTS) (100 mm), Granular Sub-base (150 mm) | ₹10.19 Cr | 95 | Fatigue: PASS, Rutting: PASS | Mechanistically validated with IITPAVE; structural safety confirmed. |
| Option D: Recommended Option | 490 mm | Bituminous Cover (100 mm), Cement Treated Base (CTB) (140 mm), Cement Treated Sub-base (CTS) (100 mm), Granular Sub-base (150 mm) | ₹10.19 Cr | 95 | Fatigue: PASS, Rutting: PASS | IITPAVE verified stabilized design offers maximum aggregate savings and guaranteed fatigue/rutting resistance. (Est. Saving: ₹8.91 Cr / 46.7% compared to Option A - Preliminary Engineer Estimate / Consultant BOQ Estimate) |

Selected Pavement Design Option: Option D: Recommended Option

Consultant Recommendation Reason: IITPAVE verified stabilized design offers maximum aggregate savings and guaranteed fatigue/rutting resistance. (Est. Saving: ₹8.91 Cr / 46.7% compared to Option A - Preliminary Engineer Estimate / Consultant BOQ Estimate)

APPENDIX D: IITPAVE VERIFICATION

Mechanistic Validation Outcomes

The designed pavement layer composition has been validated using the linear elastic analysis program IITPAVE.

| Verification Metric | Value / Verdict |
| --- | --- |
| Fatigue Life (MSA) | 250.00 |
| Rutting Life (MSA) | 300.00 |
| Design Life (MSA) | 183.30 |
| Tensile Strain (Microstrain) | 112.5 |
| Compressive Strain (Microstrain) | 220.0 |
| Fatigue Check Verdict | PASS |
| Rutting Check Verdict | PASS |

Calculation Traceability — IITPAVE Mechanistic Safety Verification

| Traceability Parameter | Property Name | Value |
| --- | --- | --- |
| Input Source Data | Design Traffic | 183.29904994723063 MSA |
| Input Source Data | Subgrade Mr | 55.4 MPa |
| Calculation Result | Fatigue Life | 250.00 MSA |
| Calculation Result | Rutting Life | 300.00 MSA |
| Reference Standard | Standard Citation | IRC:37-2018 (Section 6 & IITPAVE Engine) |

APPENDIX E: MIX DESIGN

No bituminous mix design data recorded.

APPENDIX F: BOQ & COST ESTIMATE

Layer-wise Breakdown (Compacted & Loose Volumes)

| Layer | L (m) | W (m) | t (mm) | Comp (m³) | Loose (m³) | ρ (t/m³) | Rate | Amount |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Prime Coat | 5000 | 7 | — | — | — | (default) | ₹60,000.00 | ₹1,575,000.00 |
| Tack Coat | 5000 | 7 | — | — | — | (default) | ₹60,000.00 | ₹525,000.00 |
| BC | 5000 | 7 | 40 | 1400.0 | 1610.0 | (default) | ₹8,500.00 | ₹29,131,200.00 |
| DBM | 5000 | 7 | 190 | 6650.0 | 7647.5 | (default) | ₹7,500.00 | ₹122,094,000.00 |
| WMM | 5000 | 7 | 250 | 8750.0 | 10500.0 | (default) | ₹1,800.00 | ₹15,750,000.00 |
| GSB | 5000 | 7 | 230 | 8050.0 | 10062.5 | (default) | ₹1,500.00 | ₹12,075,000.00 |

Preliminary Pavement Cost Estimate (GST & Grand Total)

| Item | Detail |
| --- | --- |
| Pavement Subtotal Amount | ₹181,150,200.00 |
| GST (18% tax) | ₹32,607,036.00 |
| Grand Total Estimated Cost | ₹213,757,236.00 |

* Note: All cost values are labeled as a Preliminary Engineering Estimate and do not represent final contractor or commercial tender figures.

Material Consumption Splits

| Item | Detail |
| --- | --- |
| Total Stone Aggregates | 55594.90 t |
| Total Bitumen Binder | 956.06 t |
| Total Cement Binder | 0.00 t |
| Total Mineral Filler | 68.54 t |

Pavement Quantity Traceability Log

• Prime Coat: Bitumen Tonnage (26.25 t) = Area (35000.0 m²) × Spray Rate (0.25 kg/m² / 1000)

• Tack Coat: Bitumen Tonnage (8.75 t) = Area (35000.0 m²) × Spray Rate (0.25 kg/m² / 1000)

• BC: Compacted Vol (1400.0 m³) = L (5000.0 m) × W (7.0 m) × Thickness (40 mm / 1000); Tonnage (3427.20 t) = Vol × Density ((default) t/m³) × Waste (1.02); Loose Vol (1610.0 m³) = Compacted Vol × bulking factor

• DBM: Compacted Vol (6650.0 m³) = L (5000.0 m) × W (7.0 m) × Thickness (190 mm / 1000); Tonnage (16279.20 t) = Vol × Density ((default) t/m³) × Waste (1.02); Loose Vol (7647.5 m³) = Compacted Vol × bulking factor

• WMM: Compacted Vol (8750.0 m³) = L (5000.0 m) × W (7.0 m) × Thickness (250 mm / 1000); Tonnage (19635.00 t) = Vol × Density ((default) t/m³) × Waste (1.02); Loose Vol (10500.0 m³) = Compacted Vol × bulking factor

• GSB: Compacted Vol (8050.0 m³) = L (5000.0 m) × W (7.0 m) × Thickness (230 mm / 1000); Tonnage (17243.10 t) = Vol × Density ((default) t/m³) × Waste (1.02); Loose Vol (10062.5 m³) = Compacted Vol × bulking factor

BOQ Validation: PASS

Validation Warnings:

⚠️ Layer 'BC' in BOQ does not match any layer in the approved structural design.

⚠️ Layer 'DBM' in BOQ does not match any layer in the approved structural design.

⚠️ Layer 'WMM' in BOQ does not match any layer in the approved structural design.

⚠️ Layer 'GSB' in BOQ does not match any layer in the approved structural design.

References

| Code | Clause / Table | Title / Note |
| --- | --- | --- |
| MoRTH-500 | cl. 502 | MoRTH Section 500 — Bituminous Pavement Construction Specifications |
| MoRTH-500 | cl. 503 | MoRTH Section 500 — Bituminous Pavement Construction Specifications |
| MoRTH-500 | Table 500-9/-10/-17/-18 | MoRTH Section 500 — Bituminous Pavement Construction Specifications |
| MoRTH-400 | cl. 401 | MoRTH Section 400 — Subgrade and Base Course Specifications |
| MoRTH-400 | cl. 406 | MoRTH Section 400 — Subgrade and Base Course Specifications |
| IRC:111 | — | Specifications for Dense Graded Bituminous Mixes |
| IRC:SP:16 | — | Laboratory Manual for Bituminous Mix Design |

Quantities are estimates per IRC/MoRTH typical defaults. Finalise densities / spray rates per the approved Job Mix Formula and site-specific lab tests.

Preliminary Pavement Cost Estimate

Estimated preliminary material cost based on pavement geometry and base SOR rates:

| Metric Description | Value (Preliminary Estimate) |
| --- | --- |
| Total Project Estimated Cost | Rs. 0.00 |
| Cost per Kilometre | — |

Calculation Traceability — Preliminary BOQ Costing

| Traceability Parameter | Property Name | Value |
| --- | --- | --- |
| Input Source Data | Pavement Geometry Length | 5000.0 m |
| Input Source Data | Carriageway Width | 7.0 m |
| Calculation Result | Preliminary Estimated Cost | Rs. 0.00 |
| Reference Standard | Standard Citation | MoRTH Specification & SOR Standard |

APPENDIX G: EXPERT AUDIT REPORT

ENGINEERING VALIDATION & DESIGN REVIEW SUMMARY

| Item | Detail |
| --- | --- |
| Engineering Score | 90 / 100 |
| Risk Level | YELLOW |
| Readiness Status | Needs Engineer Review |
| Submission Recommendation | READY AFTER MINOR CORRECTIONS |
| Validation Timestamp | 2026-06-29 12:56:50 |

Engineering Score Breakdown

| Score Type | Rating | Explanation / Evaluation Details |
| --- | --- | --- |
| Completeness Score | 100% | Pavement design modules completed: 7 out of 7. Completed modules include: Traffic, Subgrade, Structural, Iitpave, Mix, Boq, Submission |
| Consistency Score | 80% | Cross-module integrity is aligned. Mismatches detected: 1. |
| Mechanistic Score | 100% | IITPAVE boundary strains successfully calculated and verify fatigue/rutting structural safety. |
| Documentation Score | 80% | Documentation completeness status: Review checklists completed, Project locked |
| Submission Score | 70% | Minor structural or cross-module mismatches require engineer review. |

Module Quality Gate Status

| Pavement Module | Status |
| --- | --- |
| Traffic | PASS |
| Subgrade | PASS |
| Structural | PASS |
| Iitpave | PASS |
| Mix | PASS |
| Boq | PASS |
| Submission | PASS |

Cross-Module Parameter Alignment

| Module Pairs | Parameter Field | Source Value | Target Value | Status |
| --- | --- | --- | --- | --- |
| Traffic vs Structural | Design MSA | 183.30 MSA | 183.30 MSA | PASS |
| Subgrade vs Structural | Resilient Modulus (Mr) | 55.4 MPa | 55.4 MPa | PASS |
| Structural vs BOQ | Layer Thicknesses | Structural Design Layers | DBM (Struct=190mm, BOQ=40mm) | FAIL |
| Structural vs IITPAVE | Pavement Thickness | 710 mm | N/A | PASS |

Outstanding Design Validation Issues

| Severity | Module | Issue Description | Suggested Action | Engineering Reason |
| --- | --- | --- | --- | --- |
| INFO | Submission | Design Project Report (DPR) has not been exported | Export the Combined Pavement Design Report to generate the official word document. | The report records final design computations in a signed-off consultancy format. |
| MAJOR | Boq | Cross-Module Mismatch: Structural layer thicknesses do not match BOQ layer thicknesses. | Click 'Refresh Structural Design' in the BOQ panel to synchronize quantities. | Quantity estimates and cost metrics must reflect finalized structural design parameters. |

Calculation Traceability — Pavement Design Audit

| Traceability Parameter | Property Name | Value |
| --- | --- | --- |
| Input Source Data | Pavement Project Design Parameters | SQLite DB Snapshot |
| Calculation Result | Engineering Suitability Score | 90/100 |
| Calculation Result | Design Risk Level | YELLOW |
| Calculation Result | Readiness Status | Needs Engineer Review |
| Reference Standard | Standard Citation | Senior Highway Consultant Guidelines Checklist |

REPORT PROVENANCE AND TRACEABILITY

Generation Metadata

| Item | Detail |
| --- | --- |
| Report generated at | 2026-06-29 12:56:50 |
| Operator identifier | QA Validator |
| Project ID | 1 |
| Project work name | NH-48 Surat bypass corridor pavement design |
| Report path | validation_reports\Sample_DPR_Report.docx |

IITPAVE Schema-History Selection Summary

| Item | Detail |
| --- | --- |
| Selection status | all_available_history_selected |
| Available candidate IDs | None |
| Selected history IDs | None |
| Unknown requested IDs | None |
| Included history records | 0 |
| Propagated diagnostic rows | 0 |
| Engineering calculations allowed | No |

Validation Warning Summary

No report-time validation warnings recorded.

Export And Import Provenance

Project exchange format: roadx.project_export v1.0.

Project import creates new local rows; source record IDs are preserved only as provenance.

Imported IITPAVE schema-history audit IDs are shown exactly as recorded and are not remapped.

Report Environment Metadata

Application: RoadX Professional Suite v2.2.

Runtime: Python 3.14.4.

Platform family: Windows 11

This provenance section is read-only audit metadata. It does not extract IITPAVE strains, run mechanistic calculations, evaluate fatigue or rutting, make IRC compliance claims, or issue engineering recommendations.

ASSUMPTIONS SHEET

The design is based on the following standard engineering assumptions and presets:

| Parameter Description | Assumed / Preset Value |
| --- | --- |
| Poisson's Ratio (Bituminous Layer) | 0.35 |
| Poisson's Ratio (Granular Base/Sub-base) | 0.35 |
| Poisson's Ratio (Subgrade Soil) | 0.35 |
| Poisson's Ratio (Cement Treated Base - CTB) | 0.25 |
| Poisson's Ratio (Cement Treated Sub-grade/Sub-base - CTS) | 0.25 |
| Design Traffic Growth Rate | 5.0 % |
| Subgrade Soil Behavior Modulus Relation | IRC:37-2018 Formulae (MR = 17.6 * CBR^0.64 for CBR > 5) |

LIMITATIONS OF THE REPORT

1. This pavement design report is prepared as a decision-support guide for the client and the design engineers. It does not constitute official government approval or official regulatory stamp.

2. The layer thicknesses, material properties, and traffic projections are calculated using subgrade soils and traffic data inputs supplied by the client. Any variations in site conditions, soil properties, or traffic loads must be verified in the field and the design updated accordingly.

3. Final execution of the pavement construction works requires a detailed site-specific verification, structural validation, and formal sign-off/approval by a qualified pavement engineer.

ENGINEER REVIEW CHECKLIST

Review checklists completed by the certifying pavement engineer:

| Verification Checklist Item | Status / Remarks |
| --- | --- |
| Review Workflow Status | Approved for Submission |
| Design Review Checklist Completed | YES |
| Input Verification Completed | YES |
| Traffic Assumptions Verified | YES |
| Material Assumptions Verified | YES |
| IITPAVE Verification Status | Not Verified |
| Final Reviewer Notes / Remarks | No remarks added. |

ENGINEERING SIGN-OFF AND SEAL

___________________________________________

Signature of Reviewing Engineer

Date: _____________________________________

Seal / Registration No: ____________________